Emerging Data Cleaning and Fusion for Traffic Model  Calibration

User Guide for the Emerging Data Cleaning and Fusion Tool

Publication No. FHWA-HRT-25-XXX

May 2025

FOREWORD

Text here.

Name of FHWA Director, postnominal letters

Director, Office of XXX

Research and Development

Notice This document is disseminated under the sponsorship of the U.S. Department of Transportation (USDOT) in the interest of information exchange. The U.S. Government assumes no liability for the use of the information contained in this document. The U.S. Government does not endorse products or manufacturers. Trademarks or manufacturers’ names appear in this report only because they are considered essential to the objective of the document. They are included for informational purposes only and are not intended to reflect a preference, approval, or endorsement of any one product or entity. Non-Binding Contents Except for the statutes and regulations cited, the contents of this document do not have the force and effect of law and are not meant to bind the States or the public in any way. This document is intended only to provide information regarding existing requirements under the law or agency policies. Quality Assurance Statement The Federal Highway Administration (FHWA) provides high-quality information to serve Government, industry, and the public in a manner that promotes public understanding. Standards and policies are used to ensure and maximize the quality, objectivity, utility, and integrity of its information. FHWA periodically reviews quality issues and adjusts its programs and processes to ensure continuous quality improvement.

ii

TECHNICAL REPORT DOCUMENTATION PAGE

| 1. Report No. FHWA-HRT-XX-XXX | 2. Government Accession No. | 3. Recipient’s Catalog No. |
| --- | --- | --- |
| 4. Title and Subtitle Emerging Data Cleaning and |  | 5. Report Date March 2025 |

Fusion for Traffic Model Calibration: Task 3 User Guide for  the Emerging Data Cleaning and Fusion Tool

6. Performing Organization Code:

| 7. Author(s). Xuesong (Simon) Zhou, Yajun, Liu and Mustafa Gadah, Zhitong Huang, Ankur, Tyagi, Harsh Anand, and Norris K. Novat Baraba |  |  | 8. Performing Organization Report No. |  |  |
| --- | --- | --- | --- | --- | --- |
| 9. Performing Organization Name and Address Leidos Inc. 1750 Presidents St Reston, VA 20190 |  |  | 10. Work Unit No. |  |  |
|  |  |  | 11. Contract or Grant No. 693JJ321D000010-693JJ322F00274N |  |  |
| 12. Sponsoring Agency Name and Address U.S. Department of Transportation Federal Highway Administration 1200 New Jersey Avenue, SE Washington, DC 20590 |  |  | 13. Type of Report and Period Covered Draft report; December 2024 |  |  |
|  |  |  | 14. Sponsoring Agency Code HRSO |  |  |
| 15. Supplementary Notes Contraction Officer’s Representative, Jongsun Won |  |  |  |  |  |
| 16. Abstract This user guide introduces a data cleaning and fusion tool developed to support the integration of selected legacy and emerging traffic data sources. The tool is designed to enhance the accuracy and usability of traffic microsimulation models by enabling high-fidelity bottleneck identification, route choice analysis, and model calibration. It provides transportation practitioners with a practical and efficient way to process, combine, and analyze diverse data inputs. This document offers detailed instructions and reference information to help users effectively operate the tool. |  |  |  |  |  |
| 17. Key Words Data fusion, emerging data, traffic simulation, calibration, and microsimulation |  | 18. Distribution Statement No restrictions. This document is available to the public through the National Technical Information Service, Springfield, VA 22161. www.ntis.gov |  |  |  |
| 19. Security Classif. (of this report) Unclassified | 20. Security Classif. (of this page) Unclassified |  |  | 21. No. of Pages 57 | 22. Price N/A |

Form DOT F 1700.7 (8-72) Reproduction of completed page authorized.

ii

Table of Contents

EXECUTIVE SUMMARY .......................................................................................................... 7 CHAPTER 1. INTRODUCTION................................................................................................ 8

CHAPTER 2. DATA PREPARATION AND INTEGRATION FOR MODELING  OUTPUTS.................................................................................................................................... 11 Data Selection Consistent with TAT Volume III ............................................................. 11 Simulation Integration and Interoperability...................................................................... 13 GMNS in Network Preparation ........................................................................................ 13 Clarifying Route Probability Modeling and Subarea OD Flow Structures ...................... 14 Clustering Methodology for Scenario Definition ............................................................. 15 Export Network to Simulation Packages.......................................................................... 17 Guidance for Integrating Existing Simulation Networks for Data Cleaning and Fusion . 19

CHAPTER 3. INPUT FILES PREPARATION....................................................................... 21 Required Input Datasets.................................................................................................... 22 Data Folders and Relationship to Tool Submodules......................................................... 22 Folder network .................................................................................................................. 23 Folder sensor..................................................................................................................... 23 Folder tmc_speed .............................................................................................................. 24 Folder waypoint ................................................................................................................ 24 Folder trip_path ................................................................................................................ 24 Folder probe_od................................................................................................................ 25 Data Inspection and Validation with NeXTA................................................................... 26

CHAPTER 4. HOW TO USE SUBMODULES ....................................................................... 29 Download and Launch Emerging Data Cleaning and Fusion Tool .................................. 29 Optional Software Setup for Emerging Data Cleaning and Fusion Tool ......................... 31 Overview of Submodules: From Raw Input to Calibrated Output ................................... 32 Selected Study Area: The I-95 Corridor in Northern Virginia ......................................... 33 Submodule 1: Initial Data Quality Check and Formatting ............................................... 34 Submodule 2: Remove Spatial Outliers Using Map Matching......................................... 37 Submodule 3: Trend Analysis and CBI ............................................................................ 38 Submodule 4: Basic Data Fusion...................................................................................... 41 Submodule 5: Advanced Data Fusion............................................................................... 43 Submodule 6: ODME and Calibrate Initial Simulation Parameters................................. 45

LIMITATIONS........................................................................................................................... 48

APPENDIX.................................................................................................................................. 50 A. Data schema for input files: ...................................................................................... 50 B. Output file dictionary:............................................................................................... 56

REFERENCES............................................................................................................................ 58

iii

LIST OF FIGURES

Figure 1. Diagram. Integrated workflow that combines map matching, data integration,  cleaning, and analysis fusion............................................................................................... 10 Figure 2. Diagram. Impact of scenario change on subarea route choice behavior. ...................... 15 Figure 3. TMC speed data downloader by vendor C. ................................................................... 24 Figure 4. Origin-Destination analysis by vendor A platform. ...................................................... 25 Figure 5. Network verification with NeXTA................................................................................ 27 Figure 6. Route assignment visualization with NeXTA. .............................................................. 28 Figure 7. Tool right-click menu. ................................................................................................... 29 Figure 8. Properties setting. .......................................................................................................... 30 Figure 9. Map. Study area network............................................................................................... 33 Figure 10. Initial data quality check and formatting..................................................................... 35 Figure 11. Result of initial data quality check and formatting. .................................................... 35 Figure 12. Database browser......................................................................................................... 36 Figure 13. Remove spatial outliers using map matching.............................................................. 37 Figure 14. Trend analysis and CBI. .............................................................................................. 38 Figure 15. TMC-based speed profile. ........................................................................................... 39 Figure 16. End-to-end travel time................................................................................................. 40 Figure 17. Initial OD matrix heatmap........................................................................................... 40 Figure 18. Basic data fusion.......................................................................................................... 41 Figure 19. Link-based speed profile. ............................................................................................ 42 Figure 20. Advanced data fusion. ................................................................................................. 43 Figure 21. Time-dependent market penetration rate..................................................................... 44 Figure 22. Fused OD matrix heatmap........................................................................................... 44 Figure 23. ODME and calibrate initial simulation parameters. .................................................... 45 Figure 24. Time-dependent OD matrix heatmap. ......................................................................... 46 Figure 25. Fundamental diagram. ................................................................................................. 47

LIST OF TABLES

Table 1. Mapping of tool modules to TAT Volume III microsimulation steps. ........................... 12 Table 2. Checklist of network data attributes. .............................................................................. 17 Table 3. Data packages. ................................................................................................................ 22 Table 4. Input file dictionary. ....................................................................................................... 25 Table 5. Overview of submodules. ............................................................................................... 32 Table 6. Data schema for input files. ............................................................................................ 50 Table 7. Output file dictionary...................................................................................................... 56

iv

LIST OF ABBREVIATIONS

CBI congestion bottleneck identification CV connected vehicle

DOT department of transportation

FHWA federal highway administration

GIS geographic information system

GeoJSON Geographic JavaScript Object Notation GMNS general modeling network specification GPS global positioning system

LBS location-based services

MPR market penetration rate

OD origin-destination

ODME origin-destination matrix estimation OEM original equipment manufacturer

OSM OpenStreetMap

TMC traffic message channel

v

EXECUTIVE SUMMARY

This User Manual provides guidance for using the Emerging Data Cleaning and Fusion Tool,  developed under the Federal Highway Administration project titled Emerging Data Cleaning  and Fusion for Traffic Model Calibration. The tool is designed to assist transportation  professionals in processing and integrating selected traffic datasets to support improved  simulation model calibration and bottleneck analysis.

The tool addresses common challenges in working with heterogeneous traffic data, including  variations in spatial and temporal coverage, data resolution inconsistencies, and variability in  market penetration rates of connected vehicle waypoint data, trip path data, and platform-based  aggregated origin-destination (OD) data. It enables users to transform raw inputs into consistent,  simulation-ready formats through a series of structured data cleaning and fusion steps.

The underlying data fusion and calibration methodology has been documented in a separate  technical report (Zhou et al, upcoming). This open-source tool operationalizes those methods  through an integrated user interface, with built-in diagnostic messages and logging features that  allow users to inspect outputs and verify data integrity at each step of the workflow.

The six core submodules included in the tool guide users through the following key functions:

• Initial Data Quality Check and Formatting: Organizes network, sensor, waypoint and trip  path data into a standardized rational database format.

• Map Matching and Spatial Outlier Removal: Aligns trajectory and location-based data to  the underlying simulation network to improve spatial accuracy.

• Trend Analysis and Bottleneck Identification: Uses historical speed and volume data to  generate speed profiles and identify recurring congestion points.

• Basic and Advanced Data Fusion: Integrates multiple data sources to produce fused OD  matrices, speed estimates, and market penetration rates.

• OD Matrix Estimation and Initial Simulation Parameter Calibration: Produces refined OD  matrices and link-level performance indicators to support simulation and analysis.

The tool is compatible with OpenStreetMap (OSM)-derived networks and networks structured  using the General Modeling Network Specification (GMNS). The outputs produced by the tool  support congestion bottleneck analysis and the calibration of initial simulation model parameters. This manual is intended to serve as a standalone resource. While users are encouraged to consult  the companion methodology report for technical details, all procedures, inputs, and outputs  necessary to operate the tool are fully described herein.

7

CHAPTER 1. INTRODUCTION

On both state and national levels, transportation professionals are increasingly expected to justify  infrastructure investments using credible, data-driven performance measures. As congestion  mitigation continues to be a top priority, engineers and planners have turned to microsimulation  models to assess operational strategies and evaluate emerging technologies. However, preparing  reliable model inputs—including origin-destination (OD) matrices and time-dependent traffic  flow and speed profiles—has become more difficult due to the growing complexity, volume, and  diversity of available traffic data.

In recent years, state DOTs and regional agencies have gained access to a wider range of data  sources, including location-based services (LBS) data, connected vehicle (CV) waypoint  trajectories, platform-based OD aggregations, and map-matched trip path datasets, for example, aligned to OpenStreetMap (OSM) networks (OpenStreetMap contributors, 2025). While these  sources offer valuable insights into traveler behavior and network performance, they also present  integration challenges—such as inconsistent spatial and temporal resolution, data gaps, variable  market penetration rates, and the need to reconcile emerging data with legacy systems like traffic  message channel (TMC) speed records and loop detector counts.

This manual provides detailed guidance for transportation professionals to effectively utilize the  developed data cleaning and fusion methodologies developed under a Federal Highway  Administration (FHWA) project entitled “Emerging Data Cleaning and Fusion for Traffic Model Calibration”. The developed methodology aimed at enhancing the fidelity and calibration  of traffic microsimulation models by leveraging both emerging and legacy data sources. This  tool aims to equip users with the processes and techniques needed to resolve common integration  challenges, such as spatiotemporal misalignment, data quality variability, and inconsistent  coverage across sources.

For a comprehensive understanding of the methodologies and their appropriate application  contexts, users are encouraged to consult the accompanying report, Emerging Data Cleaning and  Fusion Methodology for Traffic Model Calibration, Task 3: Develop Emerging Data Cleaning  and Fusion Methodologies (Zhou, et al, upcoming). This report provides detailed technical  documentation and examples, addressing key challenges such as inconsistent spatiotemporal  resolutions, varying market penetration rates, and heterogeneous data quality. It is an essential  resource for those seeking to replicate or extend the data fusion approach within their own  modeling environments.

The Data Cleaning and Fusion Tool described in this manual equips agencies with  methodologies and steps to address these challenges systematically. Users will learn to  standardize diverse datasets, including platform-based aggregated OD data, waypoint  trajectories, trip path datasets, and TMC speed and sensor-based flow, speed, and occupancy  measurement. Users can use the tool to remove inconsistencies, fuse data across sources, and  produce reliable outputs essential for bottleneck identification and congestion analysis.

The Data Cleaning and Fusion Tool is specifically designed to:

8

• Ensure initial data quality and consistency across multiple data formats (e.g., OSM  network data, TMC speeds, sensor readings, trip paths, waypoint trajectories, and origin destination (OD) matrices).

• Resolve common issues such as spatiotemporal misalignment, variability in market  penetration rates, and spatial inaccuracies.

• Integrate LBS and CV data with traditional sensor and TMC data sources.

• Provide transportation simulation modelers with accurate, fused datasets ready for  advanced analysis and simulation calibration.

Although General Transit Feed Specification (GTFS) processing is not within the scope of this  tool, users can refer to another utility, GTFS2GMNS (Tang and Zhou, 2025), for handling GTFS  data. Additionally, incident reports are currently processed as textual log files, which are used to  map incident durations and severity levels to the underlying network, and related network-based  multimodal mobility and safety features are described in Gadah et al. (2025).

This manual is structured into several submodules as shown in Figure 1 that sequentially guide  the users through using the tool:

• Initial data quality check and formatting: standardizing and preparing datasets.

• Remove spatial outliers using map matching: aligning waypoint data to road networks to  reduce spatial inaccuracies.

• Trend analysis and congestion bottleneck identification (CBI): identifying traffic patterns  and forecasting behaviors.

• Basic data fusion: merging data from multiple sources to generate consistent speed  profiles.

• Advanced data fusion: refining traffic measures by incorporating legacy and emerging  data.

• OD matrix estimation (ODME) and calibration of initial simulation parameters:  producing robust OD matrices and calibrating simulation parameters for enhanced  modeling accuracy.

9

Source: FHWA.

Figure 1. Diagram. Integrated workflow that combines map matching, data integration,  cleaning, and analysis fusion.

This manual outlines each submodule clearly, providing explicit instructions for inputs,  processing steps, outputs, and visualization methods to ensure clarity and ease of use.

10

CHAPTER 2. DATA PREPARATION AND INTEGRATION FOR MODELING  OUTPUTS

This chapter establishes the foundation for structured data ingestion, fusion, and export by  aligning network elements and OD data with simulation requirements. It also addresses two  foundational questions central to the application of Traffic Analysis Toolbox (TAT) Volume III:  Guidelines for Applying Traffic Microsimulation Modeling Software (Wunderlich et al., 2019):

• How does the proposed analytical approach follow the TAT Volume III methodology for  selecting sufficient data for cleaning and fusion?

• How do the developed data cleaning and fusion tools align with the structure and  guidance provided in TAT Volume III?

For a detailed discussion of the rationale behind data selection, cleaning procedures, and their  role in traffic model calibration, readers are referred to the companion report: Emerging Data  Cleaning and Fusion Methodology for Traffic Model Calibration – Task 3: Framework for Data  Cleaning and Fusion (Zhou et al, upcoming).

Data Selection Consistency with TAT Volume III

Thes data selection process follows the Volume III guidance through the following steps:

• Systematic inventory of both legacy and emerging data sources. This directly applies the  TAT Volume III principle that data quality and quantity significantly influence analysis  outcomes.

• Evaluation of representativeness, spatial and temporal coverage. This step addresses  the TAT Volume III requirements to use recent and time-variant data, capturing the full  spectrum of operational conditions rather than just average scenarios.

• Mapping of available datasets to core model inputs such as OD matrices, trip paths, and  bottleneck locations. This approach follows TAT Volume III guidance on using data  from the same time period across all categories (geometric, demand, control, incident,  and weather) while tailoring data collection depth to the required analytical precision (see  Chapter 2 of the Framework Report: Emerging Data Cleaning and Fusion for Traffic  Model Calibration–Data Fusion for Microsimulation Model Calibration (Zhou et al).).

To ensure data suitability for microsimulation modeling, dataset selection must conform to the  representativeness criteria outlined in Volume III. Specifically, selected periods must adequately  reflect:

• Recurring vs. non-recurring traffic conditions.

• Weekday vs. weekend variations.

• Time-of-day traffic patterns.

11

• Month-to-month demand fluctuations.

Integration with the TAT Volume III Workflow

The tool is explicitly designed to support the multi-step calibration workflow outlined in TAT  Volume III. As summarized in Table 1, the tool complements existing analytical procedures by  preparing datasets once cluster identification has been completed externally. In alignment with

TAT Volume III guidance, it assumes that users have already conducted cluster analysis to  distinguish between conditions such as peak versus off-peak and weekday versus weekend  scenarios.

Following this initial step, the tool provides rich data preparation capabilities. These include  spatial alignment through map-matching, fusion of multiple data sources such as trip paths,  waypoints, and sensor records, and the generation of simulation-ready outputs—namely, OD  matrices, fundamental diagrams, and time-dependent path flows.

Although the tool does not conduct cluster identification itself, it is purpose-built to support  downstream data preparation steps that are consistent with the methodology and structure set  forth in Volume III.

Table 1. Mapping of tool modules to TAT Volume III microsimulation steps.

| TAT Volume III Step | Mapped Data Cleaning and Fusion Tool Submodule(s) | Key Functions and Outputs |
| --- | --- | --- |
| 1. Microsimulation Analysis Planning | Not performed by this tool | User defines scope, tool, and effort estimation based on TAT Volume III Section 1 guidance. |
| 2. Data Collection and Analysis | Not performed by this tool | Users provide data sources based on cluster analysis and coverage. |
| 3. Base Model Development | External network preprocessing tools (e.g., open-source or agency specific GIS/network builders) | Network development uses open network standard and shapefiles; downstream processing requires additional attributes for conversion to simulation-ready formats |
| 4. Error Checking | Submodule 1: Quality Check Submodule 2: Map Matching Submodule 3: Trend Analysis and CBI Submodule 4: Basic Fusion Submodule 5: Advanced Fusion | Format standardization, geospatial alignment, visualization of anomalies, consistency checks, cross-source correlation and fused data |

12

| 5. Model Calibration | Submodule 6: ODME and initial model parameter calibration. | Generates calibrated OD matrices using legacy/emerging data; supports staged calibration. |
| --- | --- | --- |
| 6. Alternatives Analysis | Not performed by this tool | Can be supported downstream using OD/path/travel time outputs generated from the tool. |
| 7. Final Report | Not performed by this tool | Report generation is outside the scope; outputs are structured for export into |

Simulation Integration and Interoperability

As part of the Base Model Development step, the proposed workflow is built upon the General  Modeling Network Specification (GMNS), which is an open, standardized format (Berg et al.,  2022) for representing transportation networks, developed to support consistent data sharing and  model interoperability. Led by the Zephyr Foundation with support from FHWA, GMNS uses  simple CSV files to define nodes, links, zones, and other network elements. It enables efficient  integration of multi-modal, multi-resolution networks into various modeling tools and supports  both static and dynamic applications. GMNS helps streamline network preparation, ensuring  compatibility across platforms and reducing duplication of effort.

GMNS defines a set of core and extended network elements—including links, nodes, zones,  centroid connectors, and turn movements—within a human- and machine-readable tabular  structure. While it does not prescribe file formats, GMNS (current version 0.96) is compatible  with CSV and SQLite formats, both used in this workflow. A detailed field dictionary, schema  structure, and evolution history are available at: https://github.com/zephyr-data-specs/GMNS.

GMNS in Network Preparation

The data preparation workflow leverages GMNS to standardize how input datasets—particularly  link.csv, node.csv, and zone.csv files from tools like OSM2GMNS are formatted and validated.  OSM2GMNS (Lu and Zhou, 2023)—is an open-source Python package that allows users to  easily extract and convert drivable networks from OpenStreetMap (OSM) into CSV files in the  GMNS format, enabling seamless data sharing and research collaboration. These standardized  inputs enable downstream data cleaning and visualization in Database Browser for SQLite (SQLite Consortium, 2025) and QGIS (Moyroud and Portet, 2018). Introduction to QGIS (QGIS  and generic tools, 1, 1-17.), while maintaining compatibility with dynamic model requirements.  Spatial topology and directional flow conventions follow GMNS principles.

As part of this pipeline:

• Input files are validated and standardized to GMNS schema.

• Spatial consistency is evaluated through GIS-based inspection, including QGIS overlay  techniques and visual validation using NeXTA (Avcı and Zhou, 2025), an open-source  platform for visualizing and editing GMNS-compliant transportation networks.

• Intermediate products are reviewed in SQLite format for consistency and schema  compliance.

13

• Cleaned network elements are directly exportable to microsimulation packages using a  checklist aligned with GMNS fields.

To use commercial simulation packages’ importing interfaces, users need to convert the CSV  files into shapefiles, a widely supported geospatial format. This conversion ensures that network  elements (such as nodes and links) have the necessary spatial geometry (e.g., points and lines)  required by these modeling platforms for proper visualization and analysis.

This chapter lays the foundation for transforming raw, heterogeneous traffic data into model ready calibration inputs, offering a data preparation workflow aligned with modern  microsimulation needs. The I-95 corridor in Northern Virginia serves as the case study to  illustrate this integration in a real-world setting. While this example focuses on a specific  corridor, the same data pipeline can be applied to other local freeway corridors. The process is  not corridor-specific; it is designed to be flexible and scalable, requiring only consistent input  data and well-defined network structures to function effectively.

This section outlines the complete setup process for the Emerging Data Cleaning and Fusion  Tool, including software installation, input data requirements, and the preparation of structured  outputs ready for simulation in modeling platforms. The goal is to guide users from raw input  ingestion to model-ready, calibrated network and demand object outputs.

State DOT simulation analysts benefit from this structure, as both legacy and emerging datasets  (TMC speed, trip paths, waypoint data) are mapped to a routable. The data source and data  package selection process is documented in the report, Emerging Data Cleaning and Fusion  Methodology for Traffic Model Calibration, Task 3: Develop Emerging Data Cleaning and  Fusion Methodologies (Zhou et al, upcoming).

In summary, GMNS is the backbone of this data integration framework, ensuring standardized  network definitions, promoting interoperability, and streamlining the transition from raw data to  calibrated traffic models.

Clarifying Route Probability Modeling and Subarea OD Flow Structures

In classical traffic assignment frameworks (e.g., static or stochastic user equilibrium), link-level  costs drive route choices, and route probabilities are derived endogenously through cost  minimization. These models treat costs (e.g., generalized travel time) as the independent variable  and solve for equilibrium path flows or selection probabilities accordingly.

However, for subarea microsimulation, particularly in commonly used microsimulation  platforms, this relationship is reversed. Route probabilities are often required as exogenous  inputs, directly fed into the model to represent traveler behavior under specified conditions.  These probabilities do not result from optimization routines but instead reflect empirically  observed behaviors, frequently derived from multi-day CV trajectory datasets.

The Data Cleaning and Fusion Tool supports both:

• Fixed-route assignments for deterministic simulation

14

• Scenario-sensitive route split generation, based on observed trajectories

While the Tool does not directly perform route choice model calibration (e.g., for logit-based  models), it generates high-quality, map-matched routes and associated travel costs that can be  used as input features for external calibration tools. This enables users to calibrate route choice  models based on empirically grounded route alternatives and travel time observations.

In subarea analysis, it is essential to define the OD relationship structure, including:

• Internal-to-Internal (I-I): Entirely within subarea; driven by internal congestion and signal  coordination.

• External-to-Internal (E-I) / Internal-to-External (I-E): Sensitive to border conditions and  demand interfaces.

• External-to-External (E-E): Often decided outside the modeled network and should be  treated as exogenous with trajectory-informed route splits.

As shown in Figure 2, route switching in E-E flows may respond to external factors not captured  within the microsimulation network. Incorporating observed route probabilities ensures these  decisions are properly embedded in the simulation inputs.

Source: FHWA.

Figure 2. Diagram. Impact of scenario change on subarea route choice behavior. Clustering Methodology for Scenario Definition

The data cleaning and fusion framework interfaces with clustering methodology at two distinct  stages—pre-processing and post-processing—serving different purposes in the microsimulation  pipeline.

1. Pre-Processing Clustering (Before Tool Execution)

15

Prior to running the Data Cleaning and Fusion Tool, users typically apply clustering methods to  legacy datasets to select the scope and time window for analysis. These upstream data sources  may include:

• Point sensor volume, occupancy and speed data

• TMC speed data

Clustering during this stage helps identify:

• Typical days for calibration (e.g., recurring weekday traffic)

• Anomalous or non-representative days to exclude

This is generally performed outside the tool and is used to define the input selection for the next  phase.

2. Fusion and Aggregation (Performed by the Tool)

The Data Cleaning and Fusion Tool ingests the selected datasets and performs:

• Format and data type standardization

• Spatial and temporal alignment

• Outlier and anomaly filtering

• Cross-source fusion

The output of the tool is a set of standardized, cleaned, and temporally aligned datasets. 3. Post-Processing Clustering (Downstream of the Tool)

Once fusion is complete, the cleaned and aligned dataset provides a high-resolution basis for  more advanced clustering and segmentation tasks, including:

• Travel behavior stratification (e.g., General Purpose Lane vs Express Lane preference  patterns)

• Dynamic OD demand segmentation (e.g., peak spread, user heterogeneity) • Scenario-sensitive behavioral shift identification (e.g., in response to tolling or incidents)

This step is typically performed by analysts or downstream modules in a modeling workflow  (e.g., OD estimation, scenario design, or machine learning-based pattern detection). The fusion  tool enables this by delivering a robust, consistent data foundation.

In summary:

• Pre-clustering selects the “when” and “what” of data for tool ingestion. • The fusion tool transforms the “raw” into “usable.”

• Post-clustering extracts behavioral insight from the “usable” data.

This architecture ensures compatibility with both legacy and emerging methodologies.

16

Export Network to Simulation Packages

After data fusion and cleaning, outputs are prepared for integration with modeling. The  following table provides a detailed checklist of essential network data attributes, including their  definitions, required validation steps, and the reasons why each check is critical. Ensuring the  accuracy and completeness of these attributes is fundamental to building a reliable and functional  microsimulation model. These checks help prevent data inconsistencies. The following Table 2  checklist ensures all required network modeling objects and attributes are properly structured.

Table 2. Checklist of network data attributes.

| Object Name | Attribute Name | Definitions | Required Check |
| --- | --- | --- | --- |
| Nodes | No | Unique identifier for each node in the network. | Ensure all nodes are uniquely numbered |
|  | XCoord, YCoord | Geographic coordinates (longitude and latitude or X/Y) of each node. | Verify coordinates are correct and within expected bounds |
|  | ControlType | Type of control at the node, such as signal, stop sign, yield, etc. | Check that control types (signals, stop signs, etc.) are correct |
|  | SpecialConditionType | Special condition type, typically for node classification (e.g., intersection, roundabout). | Ensure special conditions (e.g., intersection types) are defined |
| Links | FromNodeNo, ToNodeNo | IDs of the start and end nodes of a link. | Verify that all links have valid start and end nodes |
|  | Length | Physical length of the link, typically in meters or miles. | Check that the length is reasonable and the unit imported correctly |
|  | NumLanes | Number of lanes on the link. | Ensure lane numbers are correctly assigned |

17

| Object Name | Attribute Name | Definitions | Required Check |
| --- | --- | --- | --- |
|  | TrafficSystemCode | Traffic system code that governs allowed travel direction or modes. | Ensure that traffic system settings correctly allow traffic flow in the intended direction (important for unidirectional networks) |
|  | LinkCapacity | Link capacity per time unit (e.g., vehicles per hour). | Verify the capacity is reasonable based on link type |
|  | FreeFlowSpeed | Free-flow travel speed on the link (e.g., in km/h or mph). | Ensure free-flow speed is properly assigned (e.g., km/h) |
|  | LinkTypeCode | Link type code used for classification (e.g., freeway, arterial, local road). | Confirm the link type is correctly categorized |
| Zones | ZoneID | Unique identifier for each zone. | Ensure all zones are uniquely numbered |
|  | ZoneCode | Zone code or label for reference. | Check that zone codes are properly assigned |
|  | TypeNo | Zone classification (e.g., residential, commercial, industrial). | Verify that the zone type classification is correct |
| Centroid Connectors | FromNodeNo, ToNodeNo | Start and end nodes of centroid connectors. | Confirm that each zone is connected via a centroid connector |
|  | Length | Length of the centroid connector. | Ensure centroid connector lengths are reasonable (avoid zero-length unless necessary) |
|  | TrafficSystemCode | Traffic system code for the connector (same as for links). | Verify that centroid connectors are correctly assigned to the proper traffic system |

18

| Object Name | Attribute Name | Definitions | Required Check |
| --- | --- | --- | --- |
|  | NumLanes | Number of lanes for the connector (typically at least 1). | Check that centroid connectors have at least one lane |
| OD Matrix | Origin, Destination | Origin and destination zone identifiers. | Verify that all OD pairs are correctly assigned |
|  | Zone Connectivity | Connectivity status between zones and network. | Ensure that no demand is assigned to zones that are not connected to the network |
| Assignment Setup | Assignment Method | Selected traffic assignment method (e.g., static, dynamic, stochastic). | Verify that the correct assignment method is selected (static, dynamic, etc.) |
|  | Path Calculation | Validity of pathfinding algorithm and resulting routes. | Ensure that missing path errors are resolved |
|  | Convergence Criteria | Settings controlling when assignment iterations stop. | Check that assignment convergence settings are reasonable |

This approach ensures that cleaned and calibrated data from the Emerging Data Fusion Tool can  be integrated directly into modeling platforms, providing a seamless transition from raw inputs to  operational simulation environments.

Implementing the GMNS standard and shapefiles represents intermediate steps towards final  simulation file generation. However, transitioning from these intermediate formats to final  simulation-ready files demands considerable additional attributes and processing efforts. It's  essential to clearly highlight and systematically document these extra tasks, tailoring workflows to use-case-specific requirements and standards.

Guidance for Integrating Existing Simulation Networks for Data Cleaning and Fusion

For agencies and users who already possess established microsimulation networks—such as  those developed in a specific microscopic simulator—this section outlines how to integrate these  networks into the GMNS-compliant data cleaning and fusion pipeline. This approach enables the  application of waypoint data map matching, OD data fusion, and bottleneck identification tools  without needing to rebuild networks from scratch.

Step 1: Export Network Files with Microsimulation-Specific Attributes

19

Microsimulation networks should be exported using platform-supported data exchange formats,  such as CSV files or shapefiles, which include core network elements like nodes, links, lane  connectors, and geometric features (e.g., bend points, stop bars, and yield points). These files can  be imported into Geographic Information System (GIS) platforms to support topology  inspection, attribute validation, and spatial standardization. For example, the open-source  package vissim2gmns (Luo, 2025) converts a microsimulation network and simulated trajectory result files into Geographic JavaScript Object Notation (GeoJSON), CSV and GMNS formats for  GIS visualization and analysis. In many cases, exported networks—especially those derived  from OpenStreetMap—retain embedded OSM segment identifiers. These identifiers are critical  for aligning trajectory datasets and enabling accurate GPS waypoint map-matching during the  data fusion process.

It is important to note that shapefiles may not capture all necessary simulation attributes, such as  control logic, turn restrictions, or lane-specific details. For these cases, supplemental data should  be extracted from the simulation platform’s native configuration files, which are often stored in  XML or other structured formats. To support this, open-source utilities or community-developed  parsers may be used to convert these files into attribute-rich tabular formats compatible with GIS  tools and standardized models such as GMNS.

Step 2: Apply Cleaning and Fusion Tools

Import the converted routable network into the Emerging Data Cleaning and Fusion Tool,  then:

• Run Submodule 2 to perform CV waypoint map-matching.

• Use Submodules 4 and 5 to clean and fuse OD demand, speed data, and trip paths.

Outputs include enhanced OD matrices, travel time profiles, and bottleneck detection, linked to  identifiable network segments.

Step 3: Link Cleaned Results Back to Microsimulation Models

Use a relational database approach to bridge cleaned outputs with microsimulation model: • Create mapping tables to link standard network elements with platform-specific IDs.

• Perform attribute-based or spatial joins (e.g., via FromNodeNo, ToNodeNo, or OSM IDs)  to align fused data with existing links.

• Integrate updated OD matrices, segment-level speed, and congestion durations into  microsimulation packages via available APIs or import tools.

20

CHAPTER 3. INPUT FILES PREPARATION

This chapter introduces the required input files and their respective sources. Rather than simply  listing files, this section highlights how each dataset contributes to the broader workflow of  network fusion, demand estimation, and simulation calibration. For transportation agencies  working with both legacy and emerging data streams, a clear understanding of the structure,  challenges, and purpose of each input is essential to ensure that the outputs are reliable and  actionable.

To maintain neutrality and avoid endorsement of specific commercial products, vendor platforms  used in this analysis are referred to generically throughout the report (e.g., Vendor A:  StreetLight; Vendor A platform: StreetLight Insight; Vendor B: INRIX; Vendor C: RITIS:  Vendor D: Iteris ClearGuide platform).

Input files are organized into thematic folders (e.g., network/, sensor/, trip_path/) and must  follow a standardized structure. This enables interoperability, facilitates quality control, and  ensures smooth progression through the cleaning and fusion pipeline.

From Raw Files to Simulation-Ready Inputs

Most State DOTs already work with individual data streams like loop detector readings or third party speed profiles. However, using these datasets together—along with newer sources like trip  trajectories and platform-based OD matrices—requires careful alignment and fusion.

The tool addresses several common challenges:

• Spatial misalignment between probe data and DOT sensor coverage

• Inconsistent penetration rates in CV and LBS-derived trips

• Unmatched network references (e.g., vendor-specific segment IDs vs. GMNS links) • Overlapping or missing volumes, speeds, and paths that reduce simulation accuracy Each dataset plays a role in resolving these issues and supporting model-ready calibration: • Network topology: Foundation for map matching, routing, and OD linkage. • Sensor data: Ground truth for volume estimation and calibration validation. • TMC speed data: Captures temporal speed variability and identifies bottlenecks. • Waypoints: Supports link-level speed estimation and segment flow profiles. • Trip path data: Reveals high-resolution routing behavior and link usage patterns.

• OD matrices and vehicle routing patterns: Establish baseline demand and route  distributions for ODME and microsimulation.

21

Required Input Datasets

The tool operates on multiple data sources to support data fusion and calibration for traffic model  enhancement. Required datasets include:

• Network data: Describes the topological structure of the transportation system, including  nodes (e.g., intersections) and links (e.g., road segments). This data supports network based modeling and routing analysis.

• TMC Speed data: Time-series measurements of travel speed and travel time across  roadway segments, typically derived from traffic message channels. This data is used to  identify congestion patterns and validate model outputs.

• Sensor data: Observational data collected from fixed roadway infrastructure such as loop  detectors, radar, or camera systems. These datasets include traffic volume, speed, and  lane-level performance metrics.

• Platform-based OD data: Aggregated origin-destination matrices estimated from mobile  devices, apps, or service platforms. This data reflects observed travel demand and  supports demand model validation and adjustment.

• Emerging data packages:

o Package 1: Sequential timestamped location records from connected or GPS enabled vehicles. This data supports speed profiling, route tracking, and signal level behavior analysis.

o Package 2: Origin-to-destination trip trajectories with spatial and temporal  granularity. Useful for validating route choice behavior, estimating travel times,  and reconstructing trip patterns.

Table 3 shows the emerging data packages:

Table 3. Data packages.

| Data Types | Data Package 1 | Data Package 2 |
| --- | --- | --- |
| Emerging data type 1 | Platform-based aggregated data | Platform-based aggregated data |
| Emerging data type 2 | CV waypoint data | Trip path data |

Data Folders and Relationship to Tool Submodules

All input data must be placed into a clearly defined folder structure. This enables automated  parsing and quality checks by the tool’s internal routines.

22

Each folder corresponds to specific stages in the tool’s six-submodule pipeline (Chapter 4). This  modular alignment supports stepwise validation, map-matching, trend analysis, and OD matrix  estimation.

• Network and sensor folders support Submodule 1 (Quality Check and Formatting) • Trip path and waypoint folders support Submodules 2–5 (Map Matching and Fusion) • Probe od folder is essential for Submodule 6 (ODME and Calibration)

The alignment also supports the FHWA Traffic Analysis Toolbox Volume III microsimulation  workflow as summarized in Table 1 of the current user guide.

Folder network

The network folder contains files representing the digital road network and mapping  relationships to other data sources:

link.csv and node.csv: These two files define the network’s links and nodes, respectively. They  are generated from raw OpenStreetMap (OSM) data using the open-source OSM2GMNS tool.  You can refer to the OSM2GMNS at: https://github.com/asu-trans-ai-lab/OSM2GMNS. These

files provide the foundational topology for all subsequent mapping, matching, and traffic  analysis tasks.

tmc_to_link.csv: This file maps Traffic Message Channel (TMC) segments to network links. It  can be created using spatial join operations in GIS software such as QGIS or ArcGIS.  Alternatively, users can automate the mapping process using the open-source tmc2gmns tool. For  more information, refer to: https://github.com/asu-trans-ai-lab/MapMatching4GMNS.

zone_name_to_link.csv: This file maps probe OD zone names from vendor A to network links. It  is typically created manually by referencing zone boundaries and their overlay on the network.

SegmentId_to_link.csv: This file links vendor-specific segment identifiers (SegmentId in trip  path data) to network links. It is typically provided by the vendor B that supplies the trip path  data.

Folder sensor

The sensor folder contains sensor data provided by state DOTs:

Inventory.csv: This file contains sensor metadata, such as location, direction, and road segment  information.

lane_readings.csv: This file contains timestamped lane-level volume and occupancy readings,  which are used to estimate observed traffic volumes and to support penetration rate analysis.

23

Folder tmc_speed

The tmc_speed folder contains speed data provided by the vendor C platform, as shown in Figure  3:

TMC_Identification.csv: This file provides metadata for each TMC, including road name,  direction, and spatial extent.

Readings.csv: This file includes timestamped speed measurements associated with each TMC  segment. These readings are used to estimate traffic speeds and congestion levels.

More details can be found in the online tutorial by vendor C (RITIS, 2025).  ©RITIS.

Figure 3. TMC speed data downloader by vendor C.

Folder waypoint

The waypoint folder contains waypoint data points provided by vendor A:

waypoint.csv: This file includes timestamped coordinates associated with anonymized trips. It is  used for fine-grained map matching, link-level speed profiling, and OD flow extraction.

More details are available from the vendor A (StreetLight, 2025a), which provides a virtual panel  on connected vehicle data.

Folder trip_path

The trip_path folder contains vehicle trajectory data provided by vendor B: TripBulkReportTrajectoriesHeaders.csv: This file contains metadata about trip path.

24

trajs.csv: This file contains detailed trip path information, including time, location, speed, and  segment ID. It is used for high-resolution path analysis, OD estimation, and map matching.

More details on trip analytics data are available from the vendor B (Inrix, 2025). Folder probe_od

The probe_od folder includes OD matrix data derived from probe-based mobility sources  provided by vendor A, as shown in Figure 4:

od.csv: This file contains aggregated origin-destination trip counts between zones for a given  time period. It supports OD matrix estimation and demand calibration when fused with other  sources.

More details are available from the vendor A’s training portal (StreetLight, 2025b), and a valid  account is required.

©StreetLight.

Figure 4. Origin-Destination analysis by vendor A platform.

Table 4. Input file dictionary.

| Input File Name | Location | Description |
| --- | --- | --- |
| link.csv | input/network | Contains information about each link, including geometry, and other attributes |
| node.csv | input/network | Contains information about each node, including coordinate, and other attributes |

25

| tmc_to_link.csv | input/network | Mapping file that connects TMC segments to corresponding links |
| --- | --- | --- |
| zone_name_to_link.csv | input/network | Associates OD zone names with specific links in the network |
| SegmentId_to_link.csv | input/network | Mapping file that connects third-party Segment IDs to links |
| TMC_Identification.csv | input/tmc_speed | Contains metadata about each TMC segment (e.g., location, road name) |
| Readings.csv | input/tmc_speed | Time-stamped speed readings for each TMC segment |
| Inventory.csv | input/sensor | Inventory of all sensor information |
| lane_readings.csv | input/sensor | Detailed sensor readings |
| TripBulkReportTrajectori esHeaders.csv | input/trip_path | Header file for trajs.csv |
| trajs.csv | input/trip_path | Trip path data from vendor B |
| waypoint.csv | input/waypoint | Waypoint data from vendor A |
| od.csv | Input/od | Aggregated origin-destination matrix data from vendor A platform |

Data Inspection and Validation with NeXTA

To ensure that simulation inputs are accurate, complete, and interoperable across modeling  platforms, final network inspection using the NeXTA data hub platform (Nevers et al., 2013) is  recommended. Originally developed under the FHWA-sponsored initiative on the Effective  Integration of Analysis, Modeling, and Simulation (AMS) Tools (Nevers et al., 2013), NeXTA is  an AMS-based visualization tool (https://github.com/asu-trans-ai-lab/NeXTA4GMNS). As part  of the current FHWA project Emerging Data Cleaning and Fusion for Microsimulation Traffic  Model Calibration, NeXTA has been used to support GMNS-compliant networks. This platform  allows simulation analysts to review and validate cleaned and fused network data through an  interactive, user-friendly interface.

26

Source: FHWA.

Figure 5. Network verification with NeXTA.

NeXTA supports the following functions:

• Visual inspection of network topology, including links, nodes, zones, and centroid  connectors.

• Validation of directional flow, link-level traffic flow and speed patterns, and path-based  connectivity.

• Detection of spatial and attribute inconsistencies across OD matrices, paths, and link  volumes prior to simulation export.

• Visualization of trip path data overlaid on geospatial basemaps, allowing route-level  comparisons between major OD pairs to support quality control and model readiness.

Figures 5 and 6 illustrate example usage in NeXTA.

27

Source: FHWA.

Figure 6. Route distribution visualization with NeXTA.

28

CHAPTER 4. HOW TO USE SUBMODULES

This chapter introduces the modular architecture of the Emerging Data Cleaning and Fusion  Tool, guiding users through a structured sequence of submodules designed to transform diverse  traffic datasets into calibrated, simulation-ready outputs.

Each submodule performs a well-defined function within the larger pipeline, such as data  formatting, spatial alignment, trend analysis, data fusion, and demand calibration. These modules  can be executed sequentially, allowing users to trace the data transformation process from raw  observations (e.g., CV waypoints, sensor counts) to output files that support microsimulation  models, such as time-dependent OD matrices, link-level speed profiles, and route-level volume  patterns.

The modular structure of the tool allows users to process data in sequential steps, with  intermediate outputs and visualizations available at each stage to support quality control and  traceability. This structure allows users to:

• Inspect intermediate results for quality assurance

• Incorporate or bypass specific modules depending on available data

• Replicate and document fusion processes for different corridors or datasets Download and Launch Emerging Data Cleaning and Fusion Tool

• The tool is available through the U.S. DOT ITS Code Hub

(https://www.its.dot.gov/code/) or FHWA Saxton Lab Github (https://github.com/usdot jpo-codehub/emerging_data-cleaning-and-fusion-tool).

• After downloading, right-click the ZIP file and select “Properties”, as shown in Figure 7.

Source: FHWA.

Figure 7. Tool right-click menu.

29

• In the “Security” section at the bottom, check the box labeled “Unblock”, as shown in  Figure 8.

Source: FHWA.

Figure 8. Properties setting 1.

• Click “Apply” and then “OK” as shown in Figure 9.

Source: FHWA.

Figure 9. Properties setting 2.

30

• Unzip the file.

• Open the “Windows” folder inside the unzipped files.

• Double-click “Data Cleaning and Fusion Tool.exe” to run the tool. The tool is able to run  without the administrator privilege.

Optional Software Setup for Emerging Data Cleaning and Fusion Tool

Users can install the following optional open-source tools if database or network visualization is  needed:

• Database Browser for SQLite: Used for managing and inspecting the integrated data in  SQLite format (https://sqlitebrowser.org).

• QGIS: Supports geospatial visualization and verification of network and output layers  (https://qgis.org).

• NeXTA: Visualize and edit Analysis, Modeling, and Simulation and GMNS data sets (https://github.com/asu-trans-ai-lab/NeXTA4GMNS/tree/gh-pages)

Using QGIS Data Visualization Tools:

QGIS, an open-source GIS tool, offers a suite of data visualization tools instrumental in  preprocessing spatial data for CV waypoint and trip path analyses. Key functionalities include:

• Importing and Visualizing GMNS Files: QGIS facilitates the loading and visualization of  GMNS files with embedded geometry fields, allowing for the examination of link-based  network structures.

• Topology Correction and Layer Integration: Utilizing QGIS's topology checker, common  map-matching issues such as disconnected nodes, dangling links, or overlapping  geometries can be identified and addressed. CV waypoint data can be overlaid onto the  GMNS base network to detect spatial misalignments, gaps, or inconsistencies requiring  correction.

Using NeXTA Data Visualization Tools:

NeXTA complements QGIS by providing data visualization tools for temporal and network-level  analysis of CV data, particularly in support of microsimulation validation through the following  functionalities:

• Trajectory Visualization: Time-dependent CV trajectories are rendered to observe vehicle  paths and travel behaviors at detailed temporal resolutions.

31

• Space-Time Diagrams with CBI Tools: These diagrams capture dynamic traffic flow  patterns over time and space, assisting in the identification of queuing, shockwaves, and  recurrent congestion locations.

• Performance Metrics: NeXTA computes key performance indicators at the link level  from map-matched trajectory files, including vehicle counts, average speed, and  occupancy. These outputs can serve as benchmarks for calibrating microsimulation  models.

Overview of Submodules: From Raw Input to Calibrated Output

Each submodule in this workflow performs a specific function in the data cleaning,  transformation, fusion, or calibration pipeline. The methodology is documented in the report  Emerging Data Cleaning and Fusion Methodology for Traffic Model Calibration, Task 3:  Develop Emerging Data Cleaning and Fusion Methodologies (Zhou et al, forthcoming). These  functions range from quality assurance and spatial alignment to advanced fusion of legacy and  emerging data sources—including CV waypoint data, probe-based trip paths, and traffic-sensor  data. The output of each submodule serves as the input to the next, enabling a step-by-step  progression toward generating high-quality OD matrices, link-performance indicators, and  calibrated parameters suitable for use in microsimulation platforms. Table 5 demonstrates  overview of the inputs and outputs for each submodule.

Table 5. Overview of submodules.

| Submodules | Input Files | Output Files |
| --- | --- | --- |
| 1. Initial data quality check and formatting | link.csv node.csv tmc_to_link.csv zone_name_to_link.csv SegmentId_to_link.csv TMC_Identification.csv Readings.csv Inventory.csv lane_readings.csv TripBulkReportTrajectoriesHeaders.csv trajs.csv waypoint.csv od.csv | Unified_database.db schema.txt |
| 2. Remove spatial outliers using map matching | Waypoint data form unified_database.db | mapmatching.csv |
| 3. Trend analysis and CBI | network waypoint TMC_Identification Readings | TMC speed profile TMC CBI heatmap Corridor end-to-end travel time profile |

32

| Submodules | Input Files | Output Files |
| --- | --- | --- |
| 4. Basic data fusion | network waypoint trip path Readings | fused link speed profile fused link CBI heatmap |
| 5. Advanced data fusion | network waypoint trip path od | Penetration rate analysis OD volume fusion |
| 6. ODME and calibration of initial simulation parameters | unified_database.db | ODME result link_performance.csv route_assignment.csv Fundame n tal Diagram and Calibrated Parameters log |

Selected Study Area: The I-95 Corridor in Northern Virginia

Given the comprehensive availability of both legacy and emerging data sources accessible to the  project team, along with an existing simulation network, the I-95 corridor in Northern Virginia  (as shown in Figure 9) has been selected as the study area.

Original map: © 2024 OpenStreetMap contributors. Lines and numerical overlays added by FHWA.  Figure 10. Map. Study area network.

33

Submodule 1: Initial Data Quality Check and Formatting

Objective:

This submodule focuses on assessing the characteristics of emerging data sources while ensuring  raw data is standardized and prepared for analysis. The process involves evaluating data quality,  identifying inconsistencies, and applying necessary formatting adjustments to create a clean and  structured dataset suitable for further processing and analysis.

Process:

1. Construct the data folder:

── input

│ ── network

│ │ ├── link.csv

│ │ ├── node.csv

│ │ ├── tmc_to_link.csv

│ │ ├── zone_name_to_link.csv

│ │ ├── SegmentId_to_link.csv

│ ── sensor

│ │ ├── Inventory.csv

│ │ └── lane_readings.csv

│ ── tmc_speed

│ │ ├── TMC_Identification.csv

│ │ └── Readings.csv

│ ── trip_path

│ │ ├── TripBulkReportTrajectoriesHeaders.csv

│ │ └── trajs.csv

│ ── waypoint

│ │ └── waypoint.csv

│ ── probe_od

│ │ └── od.csv

── output

2. Run “Initial Data Quality Check and Formatting”:

• Choose your data input and output folder, and the package (Package 1 or Package 2)  you want to use, as shown in Figure 10:

34

Source: FHWA.

Figure 11. Initial data quality check and formatting.

• Click the “Initial Data Quality Check and Formatting” button, and you will see the  process information and results in the bottom-left textbox, as shown in Figure 11:

Source: FHWA.

Figure 12. Result of initial data quality check and formatting.

• The outputs are in the input/database folder, including unified database and schema. 3. Database visualization:

• Click the “DB Browser” button, and you can visualize the SQLite database. 4. Outputs:

35

• Unified_database.db:

The unified database consolidates and cleanses raw input files into a single, structured  SQLite database, ensuring consistency in data types and formats. It serves as the  foundational data store for all downstream modules, supporting integrated queries and analysis. Database Browser for SQLite can be used to visualize it, as shown in Figure  12.

• schema.txt:

This text file documents the schema of the unified database, including table names,  field types, and relational structures. It helps users understand the organization of the  data and supports extension or integration with external SQL-based platforms.

Source: FHWA.

Figure 13. Database browser.

36

Submodule 2: Remove Spatial Outliers Using Map Matching

Objective:

This submodule examines the map-matching procedure to align waypoint data with a road  network. It aims to associate each waypoint datum with its corresponding network link, ensuring  proper alignment between waypoint data and the underlying network structure.

Process:

1. Run “Remove Spatial Outliers Using Map Matching”:

• Once the “Initial Data Quality Check and Formatting” submodule is completed, the “Remove Spatial Outliers Using Map Matching” button will be unlocked. Click the  “Remove Spatial Outliers Using Map Matching” button, and you will see the process  information and results in the textbox at lower left, as shown in Figure 13:

Source: FHWA.

Figure 14. Remove spatial outliers using map matching.

• The outputs are in the output/mapmatching folder, including map matching result. 2. Outputs:

• mapmatching.csv:

This file contains the results of the map-matching process, aligning each waypoint to  a specific road network link. It also excludes spatial outliers to improve the accuracy  of subsequent analyses. The file is essential for validating waypoint trace alignment  and preparing clean input for traffic analysis or routing models.

37

Submodule 3: Trend Analysis and CBI

Objective:

This submodule is designed to identify traffic patterns and forecast future behaviors based on  historical data, as well as for congestion bottleneck identification.

Process:

1. Run “Trend Analysis and CBI”:

• Once the “Remove Spatial Outliers Using Map Matching” submodule is completed, the “Trend Analysis and CBI” button will be unlocked. Click the “Trend Analysis and  CBI” button, and you will see the process information and results in the textbox at  lower left, as shown in Figure14:

Source: FHWA.

Figure 15. Trend analysis and CBI.

• The outputs are in the output/trend_analysis_cbi folder, including TMC-based speed  profile, TMC-based CBI heatmap, end-to-end travel time and initial OD matrix  heatmap.

2. Outputs:

• TMC speed profile:

This time-series dataset presents average speeds over time for each TMC segment. It  enables users to visualize temporal traffic patterns, detect recurrent congestion, and  compare peak vs. off-peak performance across roadways.

• TMC CBI heatmap:

38

The TMC-based CBI heatmap visually identifies segments with persistent congestion  problems. This output helps transportation planners and analysts pinpoint high-impact  locations for congestion mitigation.

• Corridor end-to-end travel time:

This output summarizes travel time along defined corridors over various time periods.  It provides insights into corridor-level mobility, reliability, and variability, supporting

• Initial OD matrix heatmap:

The heatmap visualizes initial OD travel volumes derived from probe-based OD  datasets. It offers a high-level view of spatial travel demand distribution and serves as  a baseline for more refined demand modeling.

Figures 15, 16, and 17 show the example outputs.

Source: FHWA.

Figure 16. TMC-based speed profile.

39

Source: FHWA.

Figure 17. End-to-end travel time.

Source: FHWA.

Figure 18. Initial OD matrix heatmap.

40

Submodule 4: Basic Data Fusion

Objective:

This submodule integrates data from multiple sources for traffic analysis by aggregating diverse  datasets into a unified format, aligning their temporal and spatial dimensions. This ensures  consistency, resolves discrepancies across sources, and provides a seamless representation of  traffic conditions, forming a solid foundation for advanced traffic analysis tasks.

Process:

1. Run “Basic Data Fusion”:

• Once the “Trend Analysis and CBI” submodule is completed, the “Basic Data  Fusion” button will be unlocked. Click the “Basic Data Fusion” button, and you will  see the process information and results in the textbox at lower left, as shown in Figure  18:

Source: FHWA.

Figure 19. Basic data fusion.

• The outputs are in the output/basic_data_fusion folder, including link-based speed  profile and link-based CBI heatmap.

2. Outputs:

• Fused link speed profile:

This profile represents link-level average speeds, produced by combining multiple  data sources. The fused output smooths out data discrepancies and provides a more  stable and comprehensive view of traffic speeds over time.

41

• Fused link CBI heatmap:

The heatmap displays congestion bottlenecks at the link level, using the fused speed  data. It allows users to identify and monitor recurring congestion points with  improved accuracy over single-source analysis.

Figure 19 shows the example outputs.

Source: FHWA.

Figure 20. Link-based speed profile.

42

Submodule 5: Advanced Data Fusion

Objective:

This submodule integrates legacy and emerging data sources to refine traffic measures. It  enhances spatial and temporal resolution by combining historical trends with new observations,  adjusting market penetration rates, and refining estimates.

Process:

1. Run “Advanced Data Fusion”:

• Once the “Basic Data Fusion” submodule is completed, the “Advanced Data Fusion” button will be unlocked. Click the “Advanced Data Fusion” button, and you will see  the process information and results in the textbox at lower left, as shown in Figure 20:

Source: FHWA.

Figure 21. Advanced data fusion.

• The outputs are in the output/advanced_data_fusion folder, including time-dependent  market penetration rate and fused OD matrix heatmap.

2. Outputs:

• Time-dependent market penetration rate:

This file contains time-dependent estimates of probe data penetration rates across  different time intervals. It helps analysts understand the representativeness of  connected vehicle or mobile app data and supports adjustment of observed volumes to  estimate actual traffic.

• Fused OD matrix heatmap:

43

This output fuses OD matrices from various sources into a single, more reliable  dataset. It integrates historical trends with real-time observations, correcting for  biases and offering a refined view of travel demand patterns.

Figures 21, and 22 show the example outputs.

Source: FHWA.

Figure 22. Time-dependent market penetration rate.

Source: FHWA.

Figure 23. Fused OD matrix heatmap.

44

Submodule 6: ODME and Calibrate Initial Simulation Parameters

Objective:

This submodule evaluates and compares different data sources before ODME, analyzing  differences in market penetration rates and compatibility. It constructs key matrices for OD-to path, path-to-link, and travel-time mapping, enabling traffic-flow and travel-time integration.  The goal is to minimize errors and ensure accuracy, providing a baseline and additional packages  for further analysis.

Process:

1. Run “ODME and Calibrate Initial Simulation Parameters”:

• Once the “Advanced Data Fusion” submodule is completed, the “ODME and  Calibrate Initial Simulation Parameters” button will be unlocked. Click the “ODME  and Calibrate Initial Simulation Parameters” button, and you will see the process  information and results in the textbox at lower left, as shown in Figure 23:

Source: FHWA.

Figure 24. ODME and calibrate initial simulation parameters.

• The outputs are in the output/odme_simulation folder, including time-dependent OD  matrix heatmap, fundamental diagram, and ODME results.

2. Outputs:

• Time-dependent OD matrix heatmap:

45

This heatmap visualizes the refined OD demand over time, capturing both spatial and  temporal variations in traffic patterns. It integrates multiple data sources and  adjustments from penetration rate analysis to improve accuracy. Users can leverage  this output to calibrate simulation models, analyze peak travel periods, and support  scenario planning for traffic management.

• Fundamental Diagram:

The fundamental diagram plots the relationship between traffic flow, speed, and  density, forming the basis for traffic simulation calibration. It includes model  parameters such as critical density and capacity flow, used to tune the simulator.

• link_performance.csv:

This file contains performance metrics for each road link, generated during the  ODME process using DTALite or similar tools. It helps users evaluate how well the  simulated traffic matches observed patterns.

• route_assignment.csv:

This file lists the path choices and assigned volumes for each OD pair. It supports  validation of route choice behavior, network load distribution, and can be visualized  in NeXTA or other GIS platforms.

Figures 24 and 25 show the example outputs.

Source: FHWA.

Figure 25. Time-dependent OD matrix heatmap.

46

Source: FHWA.

Figure 26. Fundamental diagram.

47

LIMITATIONS

While the Emerging Data Cleaning and Fusion Tool offers a structured approach to integrating  legacy and emerging traffic datasets for microsimulation model calibration, its use is subject to  several limitations related to input formats, methodological scope, and performance constraints.

Users should be aware of these boundaries to ensure proper application and avoid  misinterpretation of results.

1. Supported Data Types and Formats

• The tool currently supports structured input files in CSV format, conforming to the  GMNS and documented schemas in Appendix A.

• It is designed to process the following data types:

o Legacy data: TMC speed data, DOT sensor counts (volume, speed, occupancy).

o Emerging data: CV waypoint data, trip path data, and platform-based OD  aggregations.

• Unstructured or non-tabular data sources (e.g., free-text incident logs, GTFS transit feeds,  image or video-based data) are not supported directly and may require pre-processing  using external tools. While ODME results can be exported in CSV format, the export

process may require user adaptation for other platforms not based on GMNS (e.g.,  converting CSVs to shapefiles, or XML-based formats).

2. Tool Scope and Methodological Constraints

• The tool focuses on data preprocessing and fusion to generate improved inputs for traffic  simulation models. It does not perform simulation or complete calibration workflows.  Additional steps are typically required to support platform-specific microsimulation  calibration procedures.

• The tool assumes that map-matching and segment-level associations are largely accurate.  However, residual errors may propagate throughout subsequent processing steps if initial  mapping is misaligned.

• Individual data sources, whether legacy or emerging, provide only a partial view of  traffic conditions. Comprehensive understanding typically requires the integration of  multiple datasets to capture spatial, temporal, and behavioral variability.

3. User Skill Requirements

• Although the tool includes structured datasets and supports use with graphical interfaces  such as QGIS and NEXTA (Avcı and Zhou, 2025), it assumes a basic familiarity with  transportation data formats (e.g., links, nodes, OD matrices). Users are expected to adhere

48

to the input schema. Extending the schema to accommodate new data fields requires  manual modification and is not dynamically supported.

4. Performance Considerations

• The tool has been tested on networks with up to hundred links and high-frequency CV  data. In the selected study area, the network for the I-95 freeway corridor in Northern  Virginia includes 113 nodes and 132 links. The CV data is approximately 55 GB.  However, performance may degrade with extremely large datasets (e.g., nationwide CV  streams over multiple months), depending on local computing resources.

• Execution time for submodules may vary significantly based on data volume, data  quality, and preprocessing needs. Users should allow sufficient runtime, particularly for  submodules involving map-matching or OD fusion for large networks.

• Memory usage may be high when processing detailed trajectory or OD datasets,  especially if run on systems with limited computer memory.

5. Generalizability

• The tool was validated using the I-95 freeway corridor in Northern Virginia. While it is  designed to be applicable to other corridors, the performance and quality of processed  and fused outputs may vary across different study areas. This variation is influenced by  the quality, resolution, and compatibility of local data sets, including data coverage,  sensor accuracy, and the consistency between data sources used for calibration and  validation.

• Users working with arterials (with complex signal timing) or transit-focused networks  may need to supplement the tool with additional procedures not included in the this tool as this report primarily focuses on uninterrupted freeway facilities where such  complexities are less prominent. For instance, users may need to convert city-level signal  timing plans into a moment-based network representation for data fusion, and estimate movement-specific or approach-level population volume inputs. These efforts should  account for factors such as effective green time, cycle length, and other characteristics  unique to interrupted facilities. Such procedures are particularly important for accurately  capturing delay dynamics under under-saturated and over-saturated conditions, which  directly affects how delays are reflected in probe-based data. CV data on arterial streets  can further provide information for microsimulation model calibration by providing  signal performance metrics such as arrivals on green and split failures. These data points  should be systematically integrated into the emerging data fusion model to align reported  probe-based delay information with underlying traffic signal behavior and volume flow  conditions. Additional examples of relevant data sources and supporting information can  be found in platforms such as Vendor D’s Signal Performance Measures (SPM) system (Iteris, 2025).

49

APPENDIX

A. Data schema for input files:

Table 6. Data schema for input files.

| File Name | Attribute | Type | Description |
| --- | --- | --- | --- |
| link.csv | link_id | INTEGER | Unique identifier for the road link |
|  | name | TEXT | Name of the road or street |
|  | osm_way_id | INTEGER | Original OpenStreetMap way ID |
|  | from_node_id | INTEGER | ID of the starting node of the link |
|  | to_node_id | INTEGER | ID of the ending node of the link |
|  | directed | INTEGER | Indicates whether the link is one-way (1) or two-way (0) |
|  | geometry | TEXT | Geometric representation of the link in formats like WKT |
|  | dir_flag | INTEGER | Directional flag used to represent link directionality |
|  | length | REAL | Length of the link in meters |
|  | facility_type | TEXT | Type of facility (e.g., freeway, arterial) |
|  | link_type | INTEGER | Classification code representing the type of link |
|  | free_speed | INTEGER | Free-flow speed of the link (mph or km/h) |
|  | free_speed_raw | TEXT | Raw or unprocessed speed information from source |
|  | lanes | INTEGER | Number of lanes on the link |
|  | capacity | INTEGER | Estimated vehicle capacity of the link per hour |
|  | allowed_uses | TEXT | Permitted vehicle types (e.g., car, truck, bus) |
|  | toll | TEXT | Indicates whether the link is tolled (Yes/No or amount) |
|  | notes | REAL | Additional notes or metadata |

50

| File Name | Attribute | Type | Description |
| --- | --- | --- | --- |
| node | name | REAL | Descriptive name or label (if any) |
| node.csv | node_id | INTEGER | Unique identifier for the node |
|  | osm_node_id | INTEGER | Original OpenStreetMap node ID |
|  | ctrl_type | TEXT | Control type at the node (e.g., signal, stop) |
|  | x_coord | REAL | X coordinate (longitude or projected) |
|  | y_coord | REAL | Y coordinate (latitude or projected) |
|  | is_boundary | INTEGER | Whether the node is a network boundary (1 = input, -1 = output, 2 = input & output) |
|  | activity_type | TEXT | Activity type (e.g., POI, transfer hub) |
|  | poi_id | REAL | Point of interest ID, if applicable |
|  | zone_id | REAL | Zone ID the node belongs to |
|  | notes | REAL | Additional notes or metadata |
| tmc_to_link.csv | tmc | TEXT | Traffic Message Channel (TMC) segment code |
|  | road_order | INTEGER | Order of the TMC along the road corridor |
|  | link_ids | TEXT | List of network link IDs matched to this TMC |
| zone_name_to_link.csv | Zone_Name | TEXT | Name of the zone (from vendor A or other vendors) |
|  | link_id | INTEGER | Associated network link ID |
|  | SegmentId | TEXT | Vendor-specific segment ID associated with the link |
| SegmentId_to_link.csv | SegmentId | TEXT | Vendor-specific segment ID associated with the link (from vendor B or other vendors) |
|  | link_id | INTEGER | Link ID in the network |
|  | osm_way_id | INTEGER | Corresponding OpenStreetMap way ID |

51

| File Name | Attribute | Type | Description |
| --- | --- | --- | --- |
| Inventory.csv | zone_id | INTEGER | Unique identifier for the sensor zone or station |
|  | display_name | TEXT | Display name of the sensor |
|  | state | TEXT | State where the sensor is located |
|  | rtmc | REAL | RTMC or operations center code |
|  | timezone | TEXT | Time zone of the sensor location |
|  | road | TEXT | Name of the road the sensor is on |
|  | direction | TEXT | Direction of travel monitored (e.g., NB, SB) |
|  | location_description | TEXT | Description of sensor |
|  | lane_type | TEXT | Type of lane being monitored |
|  | organization | TEXT | Agency or organization responsible for the sensor |
|  | detector_type | TEXT | Type of detection equipment used |
|  | latitude | REAL | Latitude of the sensor |
|  | longitude | REAL | Longitude of the sensor |
|  | bearing | REAL | Sensor orientation in degrees |
|  | default_speed | REAL | Default speed limit at the location |
|  | interval | REAL | Data collection interval in seconds |
|  | length | REAL | Length of the monitored segment |
| lane_readings.csv | zone_id | INTEGER | ID of the sensor location |
|  | lane_number | INTEGER | Lane number at the location |
|  | lane_id | INTEGER | Unique identifier for the specific lane |
|  | measurement_start | TEXT | Start time of the data collection interval |

52

| File Name | Attribute | Type | Description |
| --- | --- | --- | --- |
|  | speed | INTEGER | Measured speed in mph or km/h |
|  | volume | INTEGER | Number of vehicles recorded |
|  | occupancy | INTEGER | Percentage of time (0–99) a vehicle was detected in 60s |
|  | quality | INTEGER | Data quality flag |
|  | local_time | TEXT | Localized timestamp of the reading |
| TMC_Identification.csv | tmc | TEXT | TMC segment code |
|  | road | TEXT | Road name |
|  | direction | TEXT | Direction of travel (e.g., NB, SB) |
|  | intersection | TEXT | Closest intersection |
|  | state | TEXT | U.S. state abbreviation |
|  | county | TEXT | County name |
|  | zip | INTEGER | ZIP code |
|  | start_latitude | REAL | Starting latitude of the TMC |
|  | start_longitude | REAL | Starting longitude of the TMC |
|  | end_latitude | REAL | Ending latitude of the TMC |
|  | end_longitude | REAL | Ending longitude of the TMC |
|  | miles | REAL | Length of the TMC in miles |
|  | road_order | INTEGER | Order of TMC along a corridor |
|  | timezone_name | TEXT | Time zone name |
|  | type | TEXT | Road type classification (e.g., freeway, arterial) |
|  | country | TEXT | Country code |

53

| File Name | Attribute | Type | Description |
| --- | --- | --- | --- |
|  | active_start_date | TEXT | Start date for the TMC segment activity |
|  | active_end_date | REAL | End date for TMC segment activity |
| Readings.csv | tmc_code | TEXT | TMC segment code |
|  | measurement_tstam p | TEXT | Timestamp of measurement |
|  | speed | REAL | Measured speed |
|  | historical_average_ speed | REAL | Historical average speed |
|  | reference_speed | REAL | Reference speed (free-flow or posted) |
|  | travel_time_second s | REAL | Travel time in seconds |
|  | confidence_score | REAL | Confidence score of the observation |
|  | cvalue | REAL | Additional quality or reliability metric |
| trajs.csv | TripId | TEXT | Unique trip identifier |
|  | DeviceId | TEXT | Device identifier |
|  | ProviderId | TEXT | Vendor/provider identifier |
|  | TripTimezone | TEXT | Time zone for the trip |
|  | TrajIdx | INTEGER | Index of the trajectory point |
|  | TrajRawDistanceM | REAL | Raw trajectory distance in meters |
|  | TrajRawDurationM illis | INTEGER | Raw duration of the trip in milliseconds |
|  | SegmentId | TEXT | Vendor-specific segment ID |
|  | SegmentIdx | INTEGER | Index of the segment in the trip |
|  | LengthM | REAL | Length of the matched segment |
|  | CrossingStartOffset M | REAL | Distance from start point of segment |

54

| File Name | Attribute | Type | Description |
| --- | --- | --- | --- |
|  | CrossingEndOffset M | REAL | Distance from end point of segment |
|  | CrossingStartDateU tc | TEXT | UTC time when crossing started |
|  | CrossingEndDateUt c | TEXT | UTC time when crossing ended |
|  | CrossingSpeedKph | REAL | Speed in km/h during segment crossing |
|  | OnRoadNetworkSn apCount | INTEGER | Number of map-matched points |
|  | ErrorCodes | TEXT | Any error codes or flags |
| waypoint.csv | journey_id | TEXT | Unique trip or journey identifier |
|  | capture_time | INTEGER | Timestamp of the GPS reading (Unix format) |
|  | latitude | REAL | Latitude of the point |
|  | longitude | REAL | Longitude of the point |
|  | fuzzed_point | TEXT | Indicates if point is fuzzed/anonymized (true/false) |
|  | ignition_status | TEXT | Engine status at time of capture |
|  | heading_deg_north | REAL | Direction of movement (degrees from north) |
|  | elevation_ft | REAL | Elevation in feet |
|  | speed_mph | REAL | Speed in miles per hour |
| od.csv | Data_Periods | TEXT | Time period or range the data covers |
|  | Mode_of_Travel | TEXT | Mode of transportation (e.g., car, truck) |
|  | Origin_Zone_ID | REAL | ID of the origin zone |
|  | Origin_Zone_Name | TEXT | Name of the origin zone |
|  | Origin_Zone_Is_Pa ss_Through | TEXT | Whether origin is a pass through zone (Yes/No) |
|  | Origin_Zone_Direct ion_(degrees) | INTEGER | Direction of movement from origin zone |

55

| File Name | Attribute | Type | Description |
| --- | --- | --- | --- |
|  | Origin_Zone_is_Bi _Direction | TEXT | Whether the origin zone allows bi-directional trips |
|  | Destination_Zone_I D | REAL | ID of the destination zone |
|  | Destination_Zone_ Name | TEXT | Name of the destination zone |
|  | Destination_Zone_I s_Pass_Through | TEXT | Whether destination is a pass through zone (Yes/No) |
|  | Destination_Zone_ Direction_(degrees) | INTEGER | Direction toward destination zone |
|  | Destination_Zone_i s_Bi_Direction | TEXT | Whether the destination zone allows bi-directional trips |
|  | Day_Type | TEXT | Type of day |
|  | Day_Part | TEXT | Time of day |
|  | Average_Daily_O_ D_Traffic_(StL_Vo lume) | REAL | Average daily OD trip volume estimated by vendor A |
|  | Average_Daily_Ori gin_Zone_Traffic_( StL_Volume) | INTEGER | Total traffic volume leaving the origin zone |
|  | Average_Daily_Des tination_Zone_Traff ic_(StL_Volume) | REAL | Total traffic volume arriving at the destination zone |
|  | Avg_Travel_Time_ (sec) | REAL | Average travel time in seconds for the OD pair |

B. Output file dictionary:

Table 7. Output file dictionary.

| Output File Name | Location | Description |
| --- | --- | --- |
| Unified_database.db | output/database | Consolidated SQLite database storing integrated inputs and intermediate results |
| schema.txt | output/database | Text file documenting the database schema (table definitions) |
| mapmatching.csv | output/mapmatchin g | Output of map-matching results linking waypoints to network links |
| TMC speed profile | output/trend_analy sis_cbi | Time-series speed based on TMC data and segments |

56

| TMC CBI heatmap | output/trend_analy sis_cbi | CBI heatmap based on TMC data and segments |
| --- | --- | --- |
| Corridor end-to-end travel time profile | output/trend_analy sis_cbi | Travel time profile across corridors |
| Initial OD matrix | output/trend_analy sis_cbi | Initial OD volume data from probe-based OD data |
| fused link speed profile | output/basic_data_ fusion | Fsused speed data from TMCs and other data sources at link level |
| fused link CBI heatmap | output/basic_data_ fusion | CBI heatmap based on fused data at link level |
| Penetration rate analysis | output/advanced_d ata_fusion | Estimates of market penetration rates of waypoint data or trip path data |
| Fused OD matrix heatmap | output/advanced_d ata_fusion | Fused OD volume data from multiple sources |
| demand.csv | output/odme_simul ation/odme | Initial time-dependent OD matrix aggerated from vendor A platform |
| demand_target.csv | output/odme_simul ation/odme | Target OD matrix populated based on penetration rate adjustment |
| link_performance.csv | output/odme_simul ation/odme | Performance indicators for each link from DTALite ODME |
| route_assignment.csv | output/odme_simul ation/odme | Assigned routes and flows from DTALite ODME |
| Fundamental Diagram | output/odme_simul ation/fd | Calibrated speed, flow, and density relationship and model parameters |
| log | output | Execution logs and pipeline diagnostics |

57

REFERENCES

Avcı, C., & Zhou, X. (2025). NeXTA4GMNS: How to use NeXTA and QGIS to visualize and edit  Analysis, Modeling, and Simulation (AMS) and General Travel Network Format Specification  (GMNS) data sets. Arizona State University. Retrieved from https://github.com/asu-trans-ai lab/NeXTA4GMNS, last accessed May 19, 2025.

Berg, I., Smith, S., & Zhou, X. S. (2022). GMNS: A Specification for Sharing Routable Road  Networks (No. TRBAM-22-02127).

Gadah, M., Zhou, X., Abbasi, M., & Yellisetty, V. (2025). Traffic Flow Theory-Based Modeling  of Bike-Vehicle Interactions for Enhanced Safety and Mobility. Multimodal Transportation,  100202.

INRIX. (2024). Trip Analytics Brochure. Retrieved from https://inrix.com/wp content/uploads/2024/06/INRIX-Trip-Analytics-Brochure-for-web.pdf, last accessed May 19,  2025.

Iteris. (2025). ClearGuide. Retrieved from https://www.iteris.com/oursolutions/traffic-analytics software/clearguide, last accessed May 19, 2025.

Lu, J., & Zhou, X. S. (2023). Virtual track networks: A hierarchical modeling framework and  open-source tools for simplified and efficient connected and automated mobility (CAM) system  design based on general modeling network specification (GMNS). Transportation Research Part  C: Emerging Technologies, 153, 104223.

Luo, X. 2025. Vissim2GMNS. https://github.xyluo25/vissim2gmns, last accessed May 19, 2025. Moyroud, N., & Portet, F. (2018). Introduction to QGIS. QGIS and generic tools, 1, 1-17.

Nevers, B. L., Nguyen, K. M., Quayle, S., Zhou, X., & Taylor, J. (2013). The effective  integration of analysis, modeling, and simulation tools (No. FHWA-HRT-13-036). United  States. Federal Highway Administration. Office of Operations Research and Development.

OpenStreetMap contributors. (2025). OpenStreetMap. Retrieved from

https://www.openstreetmap.org, last accessed May 19, 2025.

RITIS. (2025). Probe Data Analytics Suite. Regional Integrated Transportation Information  System (RITIS). Retrieved from https://pda.ritis.org/suite/tutorials/?video=pcharts, last accessed  May 19, 2025.

SQLite Consortium. SQLite: Embedded SQL Database Engine. Version 3. Retrieved from  https://www.sqlite.org.

StreetLight. (2025a). Connected Vehicle Data Virtual Panel. Retrieved from  https://learn.streetlightdata.com/connected-vehicle-data-virtual-panel, last accessed May 19,  2025.

58

StreetLight. (2025b). StreetLight Training Portal. Retrieved from

https://training.streetlightdata.com, last accessed May 19, 2025.

Tang, F. and Zhou, X. 2024. GTFS2GMNS. https://github.com/itsfangtang/gtfs2gmns, last  accessed May 19, 2025.

Wunderlich, K. E., Vasudevan, M., & Wang, P. (2019). TAT volume III: guidelines for applying  traffic microsimulation modeling software 2019 update to the 2004 version (No. FHWA-HOP 18-036). United States. Federal Highway Administration.

Zhou, X., Luo, X., Abbasi, M., Huang, Z., & Tyagi, A. (2024). Emerging Data Cleaning and  Fusion for Traffic Model Calibration–Data Fusion for Microsimulation Model Calibration (No.  FHWA-HRT-24-142). United States. Federal Highway Administration.

Zhou, X., Liu, Y., Gadah, M., Huang, Z., Tyagi, A., Anand, H., & Novat Baraba, N. K (forthcoming). Emerging Data Cleaning and Fusion for Traffic Model Calibration:  Methodologies for Data Cleaning and Fusion. United States. Federal Highway Administration.

59